Parts

Tripod (assembled) link

Antennae (assembled) link

2 Stepper Motors (find torque, speed, and resolution requirements)

2 Motor Drivers

Mount (weight, degrees of freedom, 3d print?)

Torque Calc Elevation

- Antennae is modeled as a uniform 5 kg beam pivoting about its center.

- Inertia = (1/12)*m*L^2

- Torque = Ia -> a = T / I

- For NEMA 17 (Torque = 0.59 N*M): a = 1.4 rad/s^2

- For NEMA 23 (Torque = 3 N*M): a = 7.2 rad/s^2

- Estimated Static resistance = 0.5 N*m  ->  NEMA 17 would be insufficient

- Estimated Static resistance = 0.35 N*m

I believe the https://www.omc-stepperonline.com/nema-23-stepper-motor NEMA 23 stepper motor is our best option.

Stepping Resolution

For ISS tracking, the minimum observed angle change per step is 0.06 degrees.

The recommended driver for the NEMA 23 is the DM556T Driver

#### Calculation (NEMA 23 - 200 steps per revolution, DM556T Driver - 1/400 microstepping):

- Effective steps per revolution = 200 steps/rev * 400 = 80,000 steps/rev

- Degrees per step = 360 degrees / 80,000 steps = 0.0045 degrees/step

- Steps needed for 0.03 degrees = 0.06 degrees / 0.0045 degrees/step

13.333 steps

Microstepping is sufficient. Higher resolutions up to 1/25,600 are possible if needed.

Operating speed is not limiting since steps are so small. The operating angular acceleration is likely low so torque requirements depend primarily on resistance due to factors like friction.

Microcontroller

Power Supply